Annexure -A

WORK FOR RENOVATION WORK OF EXISTING STAIR BOX UPTO Top FLOOR  & 1ST FLOOR VERANDAH & PROVIDE NEW GRANAITE AT LOBBY, LNANDING, TREAD, RISER & SKIRTING UPTO 2ND FLOOR, 1ST FLOOR VERANDAH OF TAMLUK GHATAL CENTRAL CO-OPERATIVE BANK LTD, AT TAMLUK, PURBA MEDINIPUR

|  | Specification of work/ work details | Measurement (Approx.) |
| --- | --- | --- |
| 1. | Dismantling existing 125 th brick masonary partition wall on two sides and clear debris to Yje place as per instruction of authority. | 15.60 sq.m |
| 2. | Supplying fitting & fixing granites slabs – 15 mm to 18 mm thick in floor, lobby, landing, tread, riser etc. with uniform texture of approved brand, with admixture including preparing the existing surface – Area of each slab above 1.3 sq. m. |  |
| a) | Stair portion upto 2nd floor (tread, riser, landing, lobby only) | 94 sq. m |
| b) | For 1st Floor & 2nd Floor verandah portion only | 147 sq. m |
| c) | For ground floor portion (infront of main entry & step) | 40 sq.m |
| d) | Stair portion at 2nd floor (tread, riser, landing, lobby only) | 22 sq.m. |
| 3a) | Supplying fitting & fixing S.S. railing for stair | 31 r m |
| 3b) | Supplying fitting & fixing S.S. Post for stair | 1nos |
|  | Specification of granite slab to be supplied : - Combination color and type of granite : White ash & Black galaxy(south India) original - Converted color of granite is not allowed. - Variation lot &/ large Black spot &/ cracked granite sheet not allowed. |  |

N.B : 1) White ash slab will be  used in Tread portion of stair and Black galaxy(South India) will be used in Riser portion of stair.

2) In floor and lobby combination color will be used , middle part white ash slab with both side 1ft(max) border of Black  Galaxy .